| SENS-ROTATION PRO Real-Time 3D Spatial Attitude & Fallback Architecture | PAGE 1 OF 2 FYP Technical Spec |
| --- | --- |

1. Adaptive Dual-Pipeline Sensor Architecture

Smartphones differ by hardware tiers: premium devices feature a physical 3-axis Rate Gyroscope, whereas value-tier devices (such as Samsung Galaxy A06) rely solely on a 3-axis MEMS Accelerometer. To achieve universal compatibility, Sens-Rotation Pro deploys an autonomous dual-pipeline detection engine:

2. Spatial Coordinate System & The 3 Degrees of Freedom

The device adheres to a standard right-handed Cartesian frame where +X points right, +Y points toward the top edge, and +Z extends orthogonally outwards from the screen toward the user:

| Rotation | Axis | Physical Motion | Sensor Resolution Mode |
| --- | --- | --- | --- |
| Pitch (θ / β) | X-Axis (Transverse) | Tilting top up/down (elevation) | Full Gyro OR Gravity Accel (ax) |
| Roll (ϕ / γ) | Y-Axis (Longitudinal) | Tilting sideways like steering wheel | Full Gyro OR Gravity Ratio (ay / az) |
| Yaw (ψ / α) | Z-Axis (Vertical) | Twisting flat on a table (heading) | Gyro only (Locked to 0° in Fallback) |

| MATHEMATICAL FORMULATIONS & LOGIC Trigonometric Estimations, Zero-Point Tare & Tuning | PAGE 2 OF 2 Formulas & Code |
| --- | --- |

3. Mathematical Calculations & Trigonometric Derivations

| Pipeline A: Gyroscope Fusion Converts internal OS Kalman angles (radians) to degrees: • Pitch = round( β × 180° / π ) • Roll = round( γ × 180° / π ) • Yaw = round( α × 180° / π ) Tracks dynamic rotations continuously across all 3 spatial axes. | Pipeline B: Accelerometer Tilt (Samsung A06) Resolves static gravity acceleration g = [ax, ay, az]^T: • Pitch = atan2( -ax, √(ay² + az²) ) × (180° / π) • Roll = atan2( ay, az ) × (180° / π) • Yaw = 0° (Invariance limitation) The norm √(ay² + az²) isolates perpendicular gravity to prevent gimbal singularity. |
| --- | --- |

4. Tare Calibration, Normalization & UI Transformations

| Relative Zero-Point Tare Calibration | Allows user to calibrate phone on any slanted mount: Δθ = Pitch - Pitch_cal Δϕ = Roll - Roll_cal |
| --- | --- |
| Circular [-180°, +180°] Normalization | Eliminates 360° wrapping discontinuity: let val = deg % 360; if (val > 180) val -= 360; if (val < -180) val += 360; |
| Total Tilt Magnitude (L2 Norm) | Quantifies combined angular deflection from reference plane: Total Tilt = round( √(Pitch² + Roll²) ) |

5. Samsung Galaxy A06 (4GB RAM) Optimization Profile

| Optimization Metric | Configured Value | Technical & Hardware Rationale |
| --- | --- | --- |
| Update Interval | 60ms (~16.6 FPS) | Prevents JS thread lag on MediaTek Helio G85 while maintaining fluid animation. |
| Battery Budget | < 1.8% battery / hour | Eliminates excessive heating from continuous 60Hz/100Hz hardware polling. |
| Pause Suspension | 1,000,000ms throttle | Immediately suspends hardware polling on pause without unbinding listeners. |